Cautions

(Read Carefully Before you operate your machine)

CO2 Laser might cause injuries and burns if it touches the human skin for your personal safety and your machine safety please read very well before you operate your machine.

1- Do not leave the machine open when you turn it off lock it and take the key off

2- You must put wear specialized eyes protection against CO2 laser when using the machine

3- Don’t operate the machine without closing the top cover

4- I understand that laser beam is extremely dangerous if it touched directly the human skin, it might cause burns or injuries if used the wrong way.

5- Always check laser tube, laser lens and focus lens before operating, make sure that reflect and focus lens are clean, use cotton and alcohol to clean them, dirty reflect lens & focus lens will decrease the quality of cutting and engraving.

6- Always test the laser beam on a piece of wood under the laser head to make sure that the laser beam is correctly installed

7- Do not operate the laser power after you change the reflect lens until you close the top cover and make sure that the beam is going the right way

8- Always check the water pump, make sure that the water is circulating in the laser tube

9- Make sure that the water is not freezing inside the laser tube, during operation the temperature of water should be between 25 degree centigrade, make sure you maintain this level of temperature by using a thermometer.

10- Operate the machine in the range of temperate and humidity described in the technical specifications

11- Make sure the air pump is ON when you cut or engrave test the air flow by a piece of paper by facing it to the air pipe, some materials like plastic, wood, Plexiglas, leather might burn if the air pump is not working.

12- Always use the auto suction fan to take out the smoke and fume some gas coming from cutting or engraving materials might be dangerous and might affect your health.

13- Ground, Ground, Ground your machine
connect the machine to earth by a strong cable to avoid electrical chock that might destroy electrical parts in the machine.

Have a metal rod like 60 inch length and berry it like 45 inch in the ground and connect a 12-14 gauge cable from the metal rod to laser power supply earth pin and your machine body by making a hole and placing a bolt to hold the ground cable.

14- Make sure when you cutting to raise the material up from the table to avoid the reflection of laser beam on the back of material, if you are not using a honey comb table top use aluminum tiny slots to put the material on.

15- Water pump is placed inside water to pump water into the laser tube, Do no put your hand in water under any circumstances while the pump powered by AC electricity, disconnect power before hand in water.
16- It is highly dangerous to open the back cover of laser tube when the machine powered remember all the time that the input power of laser tube is DC15KV, because of high power voltage do not open it under any circumstances, unless the power is disconnected from machine.

When receiving and operating this machine you understand and agree that Global Motion technology Inc. is not responsible or liable for any personal injuries or property damage caused by miss using or wrong operating of this laser machine.