How to prepare bmp picture for engraving

- Have a picture with low light angle, recommended parallel medium light picture

- Use Photoshop to open any picture JPG, Tiff, BMP pictures

- From Photoshop menu Image -> Mode -> Gray scale

- From Photoshop menu Image -> Mode -> bitmap

- Select resolution up to 1200 Pixel/Inch, Method Halftone Screen, press ok

- Select the parameter on the screen above
Frequency means more round dots or lines in bitmap 
Angle means how are the dots or lines distributed base on angle
Shape can be select in dots, lines unit shape

- Check if the setup you selected fit the photo

- Save as BMP file then it is ready to be engraved